# 单片机使用代码

#include "board.h"

#include "version.h"

#include <string.h>

/* 报警阈值 */

#define ALARM_THRESHOLD		0.1

/**

* 功能描述：打印所有通道电压

* 入口参数：无

* 出口参数：-voltage：电压mV

* 返 回 值：-ret: 0成功，负数失败

*/

void PrintVoltageAll(double voltage[VIMAX])

{

RS485_Print("VI0X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI00], voltage[VI01], voltage[VI02], voltage[VI03]);

RS485_Print("VI1X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI10], voltage[VI11], voltage[VI12], voltage[VI13]);

RS485_Print("VI2X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI20], voltage[VI21], voltage[VI22], voltage[VI23]);

RS485_Print("VI3X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI30], voltage[VI31], voltage[VI32], voltage[VI33]);

RS485_Print("VI4X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI40], voltage[VI41], voltage[VI42], voltage[VI43]);

RS485_Print("VI5X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI50], voltage[VI51], voltage[VI52], voltage[VI53]);

RS485_Print("VI6X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI60], voltage[VI61], voltage[VI62], voltage[VI63]);

RS485_Print("VI7X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI70], voltage[VI71], voltage[VI72], voltage[VI73]);

//	printf("VI0X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI00], voltage[VI01], voltage[VI02], voltage[VI03]);

//	printf("VI1X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI10], voltage[VI11], voltage[VI12], voltage[VI13]);

//	printf("VI2X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI20], voltage[VI21], voltage[VI22], voltage[VI23]);

//	printf("VI3X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI30], voltage[VI31], voltage[VI32], voltage[VI33]);

//	printf("VI4X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI40], voltage[VI41], voltage[VI42], voltage[VI43]);

//	printf("VI5X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI50], voltage[VI51], voltage[VI52], voltage[VI53]);

//	printf("VI6X=%0.3f, %0.3f, %0.3f, %0.3f\n", voltage[VI60], voltage[VI61], voltage[VI62], voltage[VI63]);

//	printf("VI7X=%0.3f, %0.3f, %0.3f, %0.3f\n\n", voltage[VI70], voltage[VI71], voltage[VI72], voltage[VI73]);

}

/**

* 功能描述：判断是否报警

* 入口参数：无

* 出口参数：无

* 返 回 值：-ret: 0位报警、1报警

*/

int IsAlarm(double voltage[VIMAX])

{

uint8_t count = 0;

RS485_Print("Alarm Channel:");

for(uint8_t i=0; i<VIMAX; i++)

{

if(voltage[i] < ALARM_THRESHOLD)

{

count++;

RS485_Print("VI%d%d, ", i/4, i%4);

}

}

RS485_Print("\n\n");

if(count > 0)

return 1;

return 0;

}

/* 电压数据 */

double voltage[VIMAX];

/* 程序入口 */

int main(void)

{

int ret;

uint8_t AlarmFlag = 0;

while(1)

{

/* 按键切换报警是否开启 */

if(KEY_RED() == 0)

{

rt_thread_delay(100);

AlarmFlag = !AlarmFlag;

if(AlarmFlag)

{

LED_ON();

}

else

{

LED_OFF();

BEEP_OFF();

}

while(KEY_RED() == 0);

}

/* 读所有通道电压 */

ret = ADS_GetVoltageAll(voltage);

if(ret >= 0)

PrintVoltageAll(voltage);

/* 报警处理 */

if(IsAlarm(voltage))

{

if(AlarmFlag)

BEEP_ON();

}

else

{

BEEP_OFF();

}

rt_thread_delay(10);

}

}

/* Define to prevent recursive inclusion -------------------------------------*/

#ifndef __STM32F0XX_CONF_H

#define __STM32F0XX_CONF_H

/* Includes ------------------------------------------------------------------*/

/* Comment the line below to disable peripheral header file inclusion */

#include "stm32f0xx_adc.h"

#include "stm32f0xx_can.h"

#include "stm32f0xx_cec.h"

#include "stm32f0xx_crc.h"

#include "stm32f0xx_crs.h"

#include "stm32f0xx_comp.h"

#include "stm32f0xx_dac.h"

#include "stm32f0xx_dbgmcu.h"

#include "stm32f0xx_dma.h"

#include "stm32f0xx_exti.h"

#include "stm32f0xx_flash.h"

#include "stm32f0xx_gpio.h"

#include "stm32f0xx_syscfg.h"

#include "stm32f0xx_i2c.h"

#include "stm32f0xx_iwdg.h"

#include "stm32f0xx_pwr.h"

#include "stm32f0xx_rcc.h"

#include "stm32f0xx_rtc.h"

#include "stm32f0xx_spi.h"

#include "stm32f0xx_tim.h"

#include "stm32f0xx_usart.h"

#include "stm32f0xx_wwdg.h"

#include "stm32f0xx_misc.h"  /* High level functions for NVIC and SysTick (add-on to CMSIS functions) */

/* Exported types ------------------------------------------------------------*/

/* Exported constants --------------------------------------------------------*/

/* Uncomment the line below to expanse the "assert_param" macro in the

Standard Peripheral Library drivers code */

/* #define USE_FULL_ASSERT    1 */

/* Exported macro ------------------------------------------------------------*/

#ifdef  USE_FULL_ASSERT

/**

* @brief  The assert_param macro is used for function's parameters check.

* @param  expr: If expr is false, it calls assert_failed function which reports

*         the name of the source file and the source line number of the call

*         that failed. If expr is true, it returns no value.

* @retval None

*/

#define assert_param(expr) ((expr) ? (void)0 : assert_failed((uint8_t *)__FILE__, __LINE__))

/* Exported functions ------------------------------------------------------- */

void assert_failed(uint8_t* file, uint32_t line);

#else

#define assert_param(expr) ((void)0)

#endif /* USE_FULL_ASSERT */

#endif /* __STM32F0XX_CONF_H */

/************************ (C) COPYRIGHT STMicroelectronics *****END OF FILE****/

#ifndef __VERSION__H

#define __VERSION__H

/* 当前版本号 */

#define HVersion				"VCB32CH_V1.1"

#define SVersion				"V1.2"

/*

V1.0:

2025.06.25：

1.初始版本。

V1.1:

2025.10.18：

1. 硬件V1.1版本调试成功。

V1.2:

2025.10.22：

1. ADS_GAIN改为1000.1，ADS_GetVoltageAll增加校准。

*/

#endif

AI算法代码

/*

* 256-channel fault diagnosis algorithm for STM32F407ZGT6.

*

* This file follows xuqiu/开发文档.md:

*   - 8 boards, 32 voltage channels per board

*   - RS485 acquisition, UART abnormal-only output

*   - internal-Flash templates with 10-second trend templates

*   - KEY0 calibration workflow through a hardware hook

*   - board_channel output format, for example: 1_05：裂纹(92%)

*

* Hardware access is intentionally isolated behind weak hooks so the file can be

* compiled on PC and then integrated into the STM32 HAL project.

*/

#include <math.h>

#include <stdint.h>

#include <stdio.h>

#include <string.h>

#if defined(__has_include)

#if __has_include("fault_classifier.h") && __has_include("fault_classifier_data.h")

#include "fault_classifier.h"

#include "fault_classifier_data.h"

#define AI_CUBEAI_AVAILABLE            1

#endif

#endif

#ifndef AI_CUBEAI_AVAILABLE

#define AI_CUBEAI_AVAILABLE            0

#endif

#ifndef AI_WEAK

#if defined(__GNUC__)

#define AI_WEAK __attribute__((weak))

#else

#define AI_WEAK

#endif

#endif

#define AI_BOARD_COUNT                  8u   /* total boards in template (do not change) */

#define AI_ACTIVE_BOARD_COUNT           8u   /* boards physically connected */

#define AI_CHANNELS_PER_BOARD           32u

#define AI_CHANNEL_COUNT                (AI_BOARD_COUNT * AI_CHANNELS_PER_BOARD)

#define AI_TEMPLATE_COUNT               5u

#define AI_FAULT_COUNT                  4u

#define AI_HISTORY_SECONDS              10u

#define AI_CSV_SAMPLE_COUNT             100u

#define AI_CALIBRATION_SAMPLE_COUNT     100u

#define AI_FLASH_MAGIC                  0x41494654u /* AIFT */

#define AI_FLASH_VERSION                2u

#define AI_FLASH_OFFSET_HEADER          0u

#define AI_FLASH_OFFSET_TEMPLATES       256u

#define AI_RS485_REQUEST_SIZE           3u

#define AI_RS485_RESPONSE_SIZE          131u

#define AI_RS485_TIMEOUT_MS             50u  /* spec default; actual value (200u) is in ai_hw_impl.c */

#define AI_RS485_RETRY_COUNT            1u

#define AI_FAULT_THRESHOLD_PERCENT      85.0f

#define AI_WARNING_THRESHOLD_PERCENT    75.0f

#define AI_TREND_DISTANCE_THRESHOLD     0.85f

#define AI_MAX_WARNING_SECONDS          600.0f

#define AI_EPSILON                      0.000001f

#define AI_DECISION_MODE_LOGIC          0u

#define AI_DECISION_MODE_CUBEAI         1u

/* 0 = normal mode (anomaly-only output)

* 1 = acceptance/debug mode: all channels output with voltage + full detail

*     Note: 256 channels × ~60 bytes/line ≈ 15 KB per cycle; monitor may lag at 115200 */

#define AI_DEBUG_MODE                   0

typedef enum {

AI_TEMPLATE_NORMAL = 0,

AI_TEMPLATE_CRACK = 1,

AI_TEMPLATE_BREAK = 2,

AI_TEMPLATE_CORROSION = 3,

AI_TEMPLATE_BEND = 4

} AI_TemplateType;

typedef enum {

AI_FAULT_CRACK = 0,

AI_FAULT_BREAK = 1,

AI_FAULT_CORROSION = 2,

AI_FAULT_BEND = 3

} AI_FaultType;

typedef enum {

AI_STATUS_NORMAL = 0,

AI_STATUS_WARNING = 1,

AI_STATUS_FAULT = 2

} AI_Status;

typedef struct {

float mean_v;

float stddev_v;

float deviation_percent;

} AI_Features;

typedef struct {

uint8_t valid;

uint8_t reserved[3];

AI_Features point;

AI_Features trend[AI_HISTORY_SECONDS];

} AI_StateTemplate;

typedef struct {

uint32_t magic;

uint16_t version;

uint16_t channel_count;

uint16_t template_count;

uint16_t history_seconds;

uint32_t template_size;

uint32_t payload_size;

uint32_t checksum;

uint32_t reserved[8];

} AI_FlashHeader;

typedef struct {

AI_Status status;

AI_TemplateType type;

float confidence_percent;

AI_TemplateType warning_type;

float warning_confidence_percent;

int warning_minutes;

} AI_Result;

typedef struct {

float voltage_history[AI_CHANNEL_COUNT][AI_HISTORY_SECONDS];

uint8_t history_index[AI_CHANNEL_COUNT];

uint8_t history_count[AI_CHANNEL_COUNT];

uint8_t initialized;

} AI_Context;

static AI_Context g_ai;

static uint8_t    g_board_online[AI_BOARD_COUNT];

static uint8_t    g_board_fail_count[AI_BOARD_COUNT];

static uint8_t    g_ai_decision_mode = AI_DECISION_MODE_LOGIC;

static const char *const g_type_names[AI_TEMPLATE_COUNT] = {

"Normal", "Crack", "Break", "Corrosion", "Bend"

};

static const float g_weights[AI_FAULT_COUNT][3] = {

{0.70f, 0.20f, 0.10f},

{0.60f, 0.30f, 0.10f},

{0.65f, 0.25f, 0.10f},

{0.50f, 0.10f, 0.40f}

};

static const float g_simulated_fault_scale[AI_FAULT_COUNT] = {

-18.0f, -42.0f, -28.0f, -12.0f

};

int AI_CalculateFeatures(const float *samples_v,

uint16_t sample_count,

float normal_mean_v,

AI_Features *out);

int AI_Init(void);

int AI_SaveHeaderChecksum(void);

void AI_SetDecisionMode(uint8_t mode)

{

g_ai_decision_mode = (mode == AI_DECISION_MODE_CUBEAI) ?

AI_DECISION_MODE_CUBEAI :

AI_DECISION_MODE_LOGIC;

}

AI_WEAK int AI_HW_RS485_RequestBoard(uint8_t board_id, float voltage_v[AI_CHANNELS_PER_BOARD])

{

(void)board_id;

(void)voltage_v;

return -1;

}

AI_WEAK int AI_HW_ReadChannelSamples(uint16_t channel, float *samples_v, uint16_t max_count)

{

(void)channel;

(void)samples_v;

(void)max_count;

return -1;

}

AI_WEAK int AI_HW_FlashRead(uint32_t offset, void *dst, uint32_t bytes)

{

(void)offset;

(void)dst;

(void)bytes;

return -1;

}

AI_WEAK int AI_HW_FlashWrite(uint32_t offset, const void *src, uint32_t bytes)

{

(void)offset;

(void)src;

(void)bytes;

return -1;

}

AI_WEAK int AI_HW_FlashErase(void)

{

return -1;

}

AI_WEAK void AI_HW_UartWrite(const char *text)

{

(void)text;

}

AI_WEAK int AI_HW_KeyPressed(void)

{

return 0;

}

AI_WEAK uint32_t AI_HW_GetTickMs(void)

{

return 0u;

}

/* Wait up to 60 s for a KEY0 press-and-release. Uses wall-clock ticks so the

* timeout is CPU-frequency independent.  Serial prompt must be sent before

* calling so the operator knows to press the key. */

static void ai_wait_key_confirm(void)

{

uint32_t deadline;

/* Drain any bounce from a key that is still held when we enter */

deadline = AI_HW_GetTickMs() + 2000u;

while (AI_HW_KeyPressed()) {

if (AI_HW_GetTickMs() >= deadline) {

break;

}

}

/* Wait for the operator to press KEY0 (up to 60 s) */

deadline = AI_HW_GetTickMs() + 60000u;

while (!AI_HW_KeyPressed()) {

if (AI_HW_GetTickMs() >= deadline) {

return;

}

}

/* Drain bounce on release */

deadline = AI_HW_GetTickMs() + 2000u;

while (AI_HW_KeyPressed()) {

if (AI_HW_GetTickMs() >= deadline) {

break;

}

}

}

static float ai_absf(float value)

{

return value < 0.0f ? -value : value;

}

static float ai_clampf(float value, float low, float high)

{

if (value < low) {

return low;

}

if (value > high) {

return high;

}

return value;

}

static AI_TemplateType ai_fault_to_template(AI_FaultType fault)

{

return (AI_TemplateType)((uint8_t)fault + 1u);

}

static uint16_t ai_make_channel(uint8_t board_id, uint8_t board_channel)

{

return (uint16_t)(((uint16_t)(board_id - 1u) * AI_CHANNELS_PER_BOARD) + board_channel);

}

static uint8_t ai_board_id_from_channel(uint16_t channel)

{

return (uint8_t)((channel / AI_CHANNELS_PER_BOARD) + 1u);

}

static uint8_t ai_board_channel_from_channel(uint16_t channel)

{

return (uint8_t)(channel % AI_CHANNELS_PER_BOARD);

}

static uint32_t ai_checksum32_update(uint32_t hash, const void *data, uint32_t bytes)

{

const uint8_t *p = (const uint8_t *)data;

uint32_t i;

for (i = 0; i < bytes; ++i) {

hash ^= p[i];

hash *= 16777619u;

}

return hash;

}

static uint32_t ai_template_offset(uint16_t channel, AI_TemplateType type)

{

uint32_t index = ((uint32_t)channel * AI_TEMPLATE_COUNT) + (uint32_t)type;

return AI_FLASH_OFFSET_TEMPLATES + (index * (uint32_t)sizeof(AI_StateTemplate));

}

static uint32_t ai_payload_size(void)

{

return (uint32_t)AI_CHANNEL_COUNT * (uint32_t)AI_TEMPLATE_COUNT * (uint32_t)sizeof(AI_StateTemplate);

}

static int ai_valid_channel(uint16_t channel)

{

return channel < AI_CHANNEL_COUNT;

}

static int ai_valid_template(AI_TemplateType type)

{

return (uint8_t)type < AI_TEMPLATE_COUNT;

}

static int ai_read_template(uint16_t channel, AI_TemplateType type, AI_StateTemplate *templ)

{

if (!ai_valid_channel(channel) || !ai_valid_template(type) || templ == 0) {

return -1;

}

return AI_HW_FlashRead(ai_template_offset(channel, type), templ, (uint32_t)sizeof(*templ));

}

static int ai_write_template(uint16_t channel, AI_TemplateType type, const AI_StateTemplate *templ)

{

if (!ai_valid_channel(channel) || !ai_valid_template(type) || templ == 0) {

return -1;

}

return AI_HW_FlashWrite(ai_template_offset(channel, type), templ, (uint32_t)sizeof(*templ));

}

static uint32_t ai_calculate_payload_checksum(void)

{

AI_StateTemplate templ;

uint32_t hash = 2166136261u;

uint16_t channel;

uint8_t type;

for (channel = 0; channel < AI_CHANNEL_COUNT; ++channel) {

for (type = 0; type < AI_TEMPLATE_COUNT; ++type) {

if (ai_read_template(channel, (AI_TemplateType)type, &templ) != 0) {

memset(&templ, 0, sizeof(templ));

}

hash = ai_checksum32_update(hash, &templ, (uint32_t)sizeof(templ));

}

}

return hash;

}

static int ai_write_header(uint32_t checksum)

{

AI_FlashHeader header;

memset(&header, 0, sizeof(header));

header.magic = AI_FLASH_MAGIC;

header.version = AI_FLASH_VERSION;

header.channel_count = AI_CHANNEL_COUNT;

header.template_count = AI_TEMPLATE_COUNT;

header.history_seconds = AI_HISTORY_SECONDS;

header.template_size = (uint32_t)sizeof(AI_StateTemplate);

header.payload_size = ai_payload_size();

header.checksum = checksum;

return AI_HW_FlashWrite(AI_FLASH_OFFSET_HEADER, &header, (uint32_t)sizeof(header));

}

static int ai_header_valid(const AI_FlashHeader *header)

{

return header != 0 &&

header->magic == AI_FLASH_MAGIC &&

header->version == AI_FLASH_VERSION &&

header->channel_count == AI_CHANNEL_COUNT &&

header->template_count == AI_TEMPLATE_COUNT &&

header->history_seconds == AI_HISTORY_SECONDS &&

header->template_size == (uint32_t)sizeof(AI_StateTemplate) &&

header->payload_size == ai_payload_size();

}

static uint8_t ai_sum8(const uint8_t *data, uint16_t len)

{

uint16_t i;

uint8_t sum = 0u;

for (i = 0; i < len; ++i) {

sum = (uint8_t)(sum + data[i]);

}

return sum;

}

static float ai_read_float_le(const uint8_t *p)

{

union {

uint32_t u32;

float f32;

} value;

value.u32 = ((uint32_t)p[0]) |

((uint32_t)p[1] << 8) |

((uint32_t)p[2] << 16) |

((uint32_t)p[3] << 24);

return value.f32;

}

static void ai_history_push_voltage(uint16_t channel, float voltage_v)

{

uint8_t index = g_ai.history_index[channel];

g_ai.voltage_history[channel][index] = voltage_v;

g_ai.history_index[channel] = (uint8_t)((index + 1u) % AI_HISTORY_SECONDS);

if (g_ai.history_count[channel] < AI_HISTORY_SECONDS) {

g_ai.history_count[channel]++;

}

}

static float ai_history_get_voltage(uint16_t channel, uint8_t age_from_oldest)

{

uint8_t count = g_ai.history_count[channel];

uint8_t start;

if (count == 0u) {

return 0.0f;

}

if (count < AI_HISTORY_SECONDS) {

start = 0u;

} else {

start = g_ai.history_index[channel];

}

return g_ai.voltage_history[channel][(start + age_from_oldest) % AI_HISTORY_SECONDS];

}

static int ai_current_features_from_history(uint16_t channel,

const AI_StateTemplate *normal_template,

AI_Features *out)

{

float samples[AI_HISTORY_SECONDS];

uint8_t count = g_ai.history_count[channel];

uint8_t i;

if (normal_template == 0 || out == 0 || count == 0u) {

return -1;

}

for (i = 0; i < count; ++i) {

samples[i] = ai_history_get_voltage(channel, i);

}

return AI_CalculateFeatures(samples, count, normal_template->point.mean_v, out);

}

static int ai_history_features_at(uint16_t channel,

uint8_t upto_count,

const AI_StateTemplate *normal_template,

AI_Features *out)

{

float samples[AI_HISTORY_SECONDS];

uint8_t i;

if (normal_template == 0 || out == 0 || upto_count == 0u) {

return -1;

}

for (i = 0; i < upto_count; ++i) {

samples[i] = ai_history_get_voltage(channel, i);

}

return AI_CalculateFeatures(samples, upto_count, normal_template->point.mean_v, out);

}

static float ai_similarity(float current, float target, float normal)

{

float span = ai_absf(target - normal);

float err;

if (span < AI_EPSILON) {

span = ai_absf(target);

}

if (span < AI_EPSILON) {

span = 1.0f;

}

err = ai_absf(current - target) / span;

return ai_clampf(1.0f - err, 0.0f, 1.0f);

}

static float ai_feature_distance(const AI_Features *a,

const AI_Features *b,

const AI_Features *normal)

{

float d_mean = 1.0f - ai_similarity(a->mean_v, b->mean_v, normal->mean_v);

float d_stddev = 1.0f - ai_similarity(a->stddev_v, b->stddev_v, normal->stddev_v);

float d_dev = 1.0f - ai_similarity(a->deviation_percent,

b->deviation_percent,

normal->deviation_percent);

return sqrtf((d_mean * d_mean) + (d_stddev * d_stddev) + (d_dev * d_dev)) / sqrtf(3.0f);

}

#if AI_CUBEAI_AVAILABLE

AI_ALIGNED(4) static ai_u8 g_cubeai_activations[AI_FAULT_CLASSIFIER_DATA_ACTIVATION_1_SIZE];

static ai_handle g_cubeai_network = AI_HANDLE_NULL;

static uint8_t g_cubeai_ready = 0u;

static uint8_t g_cubeai_failed = 0u;

static const float g_cubeai_norm_mean[3] = {

1.29975879f, 0.14305101f, 11.96377945f

};

static const float g_cubeai_norm_std[3] = {

0.56779611f, 0.16056499f, 78.64228058f

};

static int ai_cubeai_init(void)

{

ai_handle activations[] = {

AI_HANDLE_PTR(g_cubeai_activations)

};

ai_error err;

if (g_cubeai_ready) {

return 0;

}

if (g_cubeai_failed) {

return -1;

}

err = ai_fault_classifier_create_and_init(&g_cubeai_network, activations, 0);

if (err.type != AI_ERROR_NONE) {

g_cubeai_network = AI_HANDLE_NULL;

g_cubeai_failed = 1u;

AI_HW_UartWrite("AI:CubeAI init failed, fallback logic\r\n");

return -1;

}

g_cubeai_ready = 1u;

AI_HW_UartWrite("AI:CubeAI model ready\r\n");

return 0;

}

static float ai_softmax_confidence(const float *logits, uint8_t index, uint8_t count)

{

float max_logit = logits[0];

float sum = 0.0f;

uint8_t i;

for (i = 1u; i < count; ++i) {

if (logits[i] > max_logit) {

max_logit = logits[i];

}

}

for (i = 0u; i < count; ++i) {

sum += expf(logits[i] - max_logit);

}

if (sum < AI_EPSILON) {

return 0.0f;

}

return (expf(logits[index] - max_logit) / sum) * 100.0f;

}

static int ai_cubeai_evaluate(const AI_Features *current, AI_Result *result)

{

float input_data[AI_FAULT_CLASSIFIER_IN_1_SIZE];

float output_data[AI_FAULT_CLASSIFIER_OUT_1_SIZE];

ai_buffer *input;

ai_buffer *output;

ai_i32 batch;

uint8_t best = 0u;

uint8_t i;

if (current == 0 || result == 0) {

return -1;

}

if (ai_cubeai_init() != 0) {

return -2;

}

input_data[0] = (current->mean_v - g_cubeai_norm_mean[0]) / g_cubeai_norm_std[0];

input_data[1] = (current->stddev_v - g_cubeai_norm_mean[1]) / g_cubeai_norm_std[1];

input_data[2] = (current->deviation_percent - g_cubeai_norm_mean[2]) / g_cubeai_norm_std[2];

input = ai_fault_classifier_inputs_get(g_cubeai_network, 0);

output = ai_fault_classifier_outputs_get(g_cubeai_network, 0);

if (input == 0 || output == 0) {

return -3;

}

input[0].data = AI_HANDLE_PTR(input_data);

output[0].data = AI_HANDLE_PTR(output_data);

batch = ai_fault_classifier_run(g_cubeai_network, input, output);

if (batch != 1) {

return -4;

}

for (i = 1u; i < AI_FAULT_CLASSIFIER_OUT_1_SIZE; ++i) {

if (output_data[i] > output_data[best]) {

best = i;

}

}

result->type = (AI_TemplateType)best;

result->confidence_percent = ai_clampf(

ai_softmax_confidence(output_data, best, AI_FAULT_CLASSIFIER_OUT_1_SIZE),

0.0f,

100.0f);

result->status = AI_STATUS_NORMAL;

if (best != (uint8_t)AI_TEMPLATE_NORMAL &&

result->confidence_percent >= AI_FAULT_THRESHOLD_PERCENT) {

result->status = AI_STATUS_FAULT;

} else if (best != (uint8_t)AI_TEMPLATE_NORMAL &&

result->confidence_percent >= AI_WARNING_THRESHOLD_PERCENT) {

result->status = AI_STATUS_WARNING;

}

return 0;

}

#else

static int ai_cubeai_evaluate(const AI_Features *current, AI_Result *result)

{

(void)current;

(void)result;

return -1;

}

#endif

static float ai_trend_distance(uint16_t channel,

const AI_StateTemplate *fault_template,

const AI_StateTemplate *normal_template)

{

AI_Features current_trend[AI_HISTORY_SECONDS];

float sum = 0.0f;

uint8_t i;

if (g_ai.history_count[channel] < AI_HISTORY_SECONDS) {

return 999.0f;

}

for (i = 0; i < AI_HISTORY_SECONDS; ++i) {

if (ai_history_features_at(channel, (uint8_t)(i + 1u), normal_template, &current_trend[i]) != 0) {

return 999.0f;

}

sum += ai_feature_distance(&current_trend[i], &fault_template->trend[i], &normal_template->point);

}

return sum / (float)AI_HISTORY_SECONDS;

}

static int ai_predict_warning(uint16_t channel,

const AI_Features *current,

const AI_StateTemplate *normal_template,

AI_TemplateType *warning_type,

float *confidence_percent,

int *minutes)

{

AI_StateTemplate fault_template;

float best_distance = 999.0f;

AI_TemplateType best_type = AI_TEMPLATE_NORMAL;

uint8_t i;

*warning_type = AI_TEMPLATE_NORMAL;

*confidence_percent = 0.0f;

*minutes = 0;

if (g_ai.history_count[channel] < AI_HISTORY_SECONDS) {

return 0;

}

for (i = 0; i < AI_FAULT_COUNT; ++i) {

AI_TemplateType type = ai_fault_to_template((AI_FaultType)i);

float distance;

if (ai_read_template(channel, type, &fault_template) != 0 || !fault_template.valid) {

continue;

}

distance = ai_trend_distance(channel, &fault_template, normal_template);

if (distance < best_distance) {

best_distance = distance;

best_type = type;

}

}

if (best_type != AI_TEMPLATE_NORMAL && best_distance < AI_TREND_DISTANCE_THRESHOLD) {

AI_Features oldest_feature;

float current_dev = current->deviation_percent;

float template_dev;

float rate_per_second;

float remaining_seconds;

if (ai_read_template(channel, best_type, &fault_template) != 0 || !fault_template.valid) {

return 0;

}

if (ai_history_features_at(channel, 1u, normal_template, &oldest_feature) != 0) {

return 0;

}

template_dev = fault_template.point.deviation_percent;

rate_per_second = (current_dev - oldest_feature.deviation_percent) / (float)AI_HISTORY_SECONDS;

if (rate_per_second >= -AI_EPSILON) {

return 0;

}

remaining_seconds = (template_dev - current_dev) / rate_per_second;

if (remaining_seconds > 0.0f && remaining_seconds < AI_MAX_WARNING_SECONDS) {

*confidence_percent = (1.0f - (best_distance / AI_TREND_DISTANCE_THRESHOLD)) * 100.0f;

*confidence_percent = ai_clampf(*confidence_percent, 0.0f, 100.0f);

*warning_type = best_type;

*minutes = (int)((remaining_seconds / 60.0f) + 0.5f);

if (*minutes < 1) {

*minutes = 1;

}

return 1;

}

}

return 0;

}

int AI_CalculateFeatures(const float *samples_v,

uint16_t sample_count,

float normal_mean_v,

AI_Features *out)

{

float sum = 0.0f;

float variance = 0.0f;

uint16_t i;

if (samples_v == 0 || out == 0 || sample_count == 0u) {

return -1;

}

for (i = 0; i < sample_count; ++i) {

sum += samples_v[i];

}

out->mean_v = sum / (float)sample_count;

for (i = 0; i < sample_count; ++i) {

float delta = samples_v[i] - out->mean_v;

variance += delta * delta;

}

out->stddev_v = sqrtf(variance / (float)sample_count);

if (ai_absf(normal_mean_v) < AI_EPSILON) {

out->deviation_percent = ai_absf(out->mean_v) < AI_EPSILON ? 0.0f : 100.0f;

} else {

out->deviation_percent = ((out->mean_v - normal_mean_v) / normal_mean_v) * 100.0f;

}

return 0;

}

int AI_RS485_BuildReadRequest(uint8_t board_id, uint8_t out_frame[AI_RS485_REQUEST_SIZE])

{

if (board_id < 1u || board_id > AI_BOARD_COUNT || out_frame == 0) {

return -1;

}

out_frame[0] = board_id;

out_frame[1] = 0x00u;

out_frame[2] = ai_sum8(out_frame, 2u);

return 0;

}

int AI_RS485_ParseReadResponse(uint8_t board_id,

const uint8_t frame[AI_RS485_RESPONSE_SIZE],

float voltage_v[AI_CHANNELS_PER_BOARD])

{

uint8_t ch;

if (board_id < 1u || board_id > AI_BOARD_COUNT || frame == 0 || voltage_v == 0) {

return -1;

}

if (frame[0] != board_id || frame[1] != 0x01u) {

return -2;

}

if (ai_sum8(frame, (uint16_t)(AI_RS485_RESPONSE_SIZE - 1u)) != frame[AI_RS485_RESPONSE_SIZE - 1u]) {

return -3;

}

for (ch = 0; ch < AI_CHANNELS_PER_BOARD; ++ch) {

voltage_v[ch] = ai_read_float_le(&frame[2u + ((uint16_t)ch * 4u)]);

}

return 0;

}

static void ai_scan_boards(void)

{

uint8_t  id;

float    dummy[AI_CHANNELS_PER_BOARD];

char     buf[32];

/* Wait for acquisition boards to finish their own power-on sequence */

AI_HW_UartWrite("SCAN:waiting for boards...\r\n");

{

uint32_t t = AI_HW_GetTickMs() + 1000u;

while (AI_HW_GetTickMs() < t) { /* busy-wait 1 s */ }

}

AI_HW_UartWrite("SCAN:checking boards...\r\n");

for (id = 1u; id <= AI_ACTIVE_BOARD_COUNT; ++id) {

int rc = AI_HW_RS485_RequestBoard(id, dummy);

g_board_online[id - 1u] = (rc == 0) ? 1u : 0u;

snprintf(buf, sizeof(buf), "Board %u:%s\r\n",

(unsigned)id, rc == 0 ? "OK" : "TIMEOUT");

AI_HW_UartWrite(buf);

}

}

int AI_Init(void)

{

AI_FlashHeader header;

memset(&g_ai, 0, sizeof(g_ai));

memset(g_board_online, 0, sizeof(g_board_online));

memset(g_board_fail_count, 0, sizeof(g_board_fail_count));

if (AI_HW_FlashRead(AI_FLASH_OFFSET_HEADER, &header, (uint32_t)sizeof(header)) != 0 ||

!ai_header_valid(&header)) {

AI_HW_UartWrite("AI:Flash template invalid\r\n");

g_ai.initialized = 1u;

return -1;

}

if (header.checksum != 0u && header.checksum != ai_calculate_payload_checksum()) {

AI_HW_UartWrite("AI:Flash checksum error\r\n");

g_ai.initialized = 1u;

return -2;

}

AI_HW_UartWrite("AI:Templates OK\r\n");

g_ai.initialized = 1u;

ai_scan_boards();

return 0;

}

int AI_EraseTemplates(void)

{

memset(&g_ai, 0, sizeof(g_ai));

if (AI_HW_FlashErase() != 0) {

return -1;

}

return ai_write_header(0u);

}

int AI_SaveHeaderChecksum(void)

{

return ai_write_header(ai_calculate_payload_checksum());

}

int AI_TemplateFromSamples(uint16_t channel,

AI_TemplateType type,

const float *samples_v,

uint16_t sample_count,

AI_StateTemplate *out)

{

AI_StateTemplate normal_template;

float normal_mean = 0.0f;

uint8_t i;

if (!ai_valid_channel(channel) || !ai_valid_template(type) ||

samples_v == 0 || out == 0 || sample_count == 0u) {

return -1;

}

memset(out, 0, sizeof(*out));

if (type != AI_TEMPLATE_NORMAL) {

if (ai_read_template(channel, AI_TEMPLATE_NORMAL, &normal_template) != 0 ||

!normal_template.valid) {

return -2;

}

normal_mean = normal_template.point.mean_v;

}

if (AI_CalculateFeatures(samples_v, sample_count, normal_mean, &out->point) != 0) {

return -3;

}

if (type == AI_TEMPLATE_NORMAL) {

normal_mean = out->point.mean_v;

out->point.deviation_percent = 0.0f;

}

for (i = 0; i < AI_HISTORY_SECONDS; ++i) {

uint16_t len = (uint16_t)(i + 1u);

uint16_t start = sample_count > AI_HISTORY_SECONDS ? (uint16_t)(sample_count - AI_HISTORY_SECONDS) : 0u;

if ((uint16_t)(start + len) > sample_count) {

len = (uint16_t)(sample_count - start);

}

(void)AI_CalculateFeatures(&samples_v[start], len, normal_mean, &out->trend[i]);

if (type == AI_TEMPLATE_NORMAL) {

out->trend[i].deviation_percent = 0.0f;

}

}

out->valid = 1u;

return 0;

}

int AI_StoreTemplateFromSamples(uint16_t channel,

AI_TemplateType type,

const float *samples_v,

uint16_t sample_count)

{

AI_StateTemplate templ;

int rc = AI_TemplateFromSamples(channel, type, samples_v, sample_count, &templ);

if (rc != 0) {

return rc;

}

return ai_write_template(channel, type, &templ);

}

int AI_GenerateSimulatedFaultTemplates(void)

{

AI_StateTemplate normal_template;

AI_StateTemplate fault_template;

uint16_t channel;

uint8_t fault;

uint8_t i;

for (channel = 0; channel < AI_CHANNEL_COUNT; ++channel) {

if (ai_read_template(channel, AI_TEMPLATE_NORMAL, &normal_template) != 0 ||

!normal_template.valid) {

continue;

}

for (fault = 0; fault < AI_FAULT_COUNT; ++fault) {

float scale = g_simulated_fault_scale[fault];

memset(&fault_template, 0, sizeof(fault_template));

fault_template.valid = 1u;

fault_template.point = normal_template.point;

fault_template.point.mean_v = normal_template.point.mean_v * (1.0f + (scale / 100.0f));

fault_template.point.stddev_v = normal_template.point.stddev_v + ai_absf(normal_template.point.mean_v * scale * 0.002f);

fault_template.point.deviation_percent = scale;

for (i = 0; i < AI_HISTORY_SECONDS; ++i) {

float ratio = (float)(i + 1u) / (float)AI_HISTORY_SECONDS;

fault_template.trend[i] = normal_template.point;

fault_template.trend[i].mean_v = normal_template.point.mean_v * (1.0f + ((scale * ratio) / 100.0f));

fault_template.trend[i].stddev_v = normal_template.point.stddev_v +

ai_absf(normal_template.point.mean_v * scale * ratio * 0.002f);

fault_template.trend[i].deviation_percent = scale * ratio;

}

if (ai_write_template(channel, ai_fault_to_template((AI_FaultType)fault), &fault_template) != 0) {

return -1;

}

}

}

return AI_SaveHeaderChecksum();

}

int AI_CalibrateChannel(uint16_t channel, AI_TemplateType type)

{

float samples[AI_CALIBRATION_SAMPLE_COUNT];

int count;

if (!g_ai.initialized) {

(void)AI_Init();

}

if (!ai_valid_channel(channel) || !ai_valid_template(type)) {

return -1;

}

count = AI_HW_ReadChannelSamples(channel, samples, AI_CALIBRATION_SAMPLE_COUNT);

if (count <= 0) {

return -2;

}

return AI_StoreTemplateFromSamples(channel, type, samples, (uint16_t)count);

}

int AI_CalibrateAll(AI_TemplateType type)

{

uint16_t channel;

uint16_t active_channels = (uint16_t)(AI_ACTIVE_BOARD_COUNT * AI_CHANNELS_PER_BOARD);

for (channel = 0; channel < active_channels; ++channel) {

(void)AI_CalibrateChannel(channel, type);   /* skip offline channels, don't abort */

}

return AI_SaveHeaderChecksum();

}

int AI_EvaluateChannel(uint16_t channel, const AI_Features *current, AI_Result *result)

{

AI_StateTemplate normal_template;

AI_StateTemplate fault_template;

float best_score = 0.0f;

AI_TemplateType best_type = AI_TEMPLATE_NORMAL;

uint8_t use_logic = 1u;

uint8_t i;

if (!ai_valid_channel(channel) || current == 0 || result == 0) {

return -1;

}

memset(result, 0, sizeof(*result));

result->type = AI_TEMPLATE_NORMAL;

result->warning_type = AI_TEMPLATE_NORMAL;

result->status = AI_STATUS_NORMAL;

if (ai_read_template(channel, AI_TEMPLATE_NORMAL, &normal_template) != 0 ||

!normal_template.valid) {

return -2;

}

if (g_ai_decision_mode == AI_DECISION_MODE_CUBEAI) {

if (ai_cubeai_evaluate(current, result) == 0) {

use_logic = 0u;

}

}

if (use_logic) {

for (i = 0; i < AI_FAULT_COUNT; ++i) {

AI_TemplateType type = ai_fault_to_template((AI_FaultType)i);

const float *weight = g_weights[i];

float mean_score;

float stddev_score;

float deviation_score;

float score;

if (ai_read_template(channel, type, &fault_template) != 0 || !fault_template.valid) {

continue;

}

mean_score = ai_similarity(current->mean_v,

fault_template.point.mean_v,

normal_template.point.mean_v);

stddev_score = ai_similarity(current->stddev_v,

fault_template.point.stddev_v,

normal_template.point.stddev_v);

deviation_score = ai_similarity(current->deviation_percent,                                  fault_template.point.deviation_percent,                                          normal_template.point.deviation_percent);

score = ((mean_score * weight[0]) +

(stddev_score * weight[1]) +

(deviation_score * weight[2])) * 100.0f;

if (score > best_score) {

best_score = score;

best_type = type;

}

}

result->type = best_type;

result->confidence_percent = ai_clampf(best_score, 0.0f, 100.0f);

if (best_type != AI_TEMPLATE_NORMAL && result->confidence_percent >= AI_FAULT_THRESHOLD_PERCENT) {

result->status = AI_STATUS_FAULT;

} else if (best_type != AI_TEMPLATE_NORMAL &&

result->confidence_percent >= AI_WARNING_THRESHOLD_PERCENT) {

result->status = AI_STATUS_WARNING;

}

}

if (ai_predict_warning(channel,

current,

&normal_template,

&result->warning_type,

&result->warning_confidence_percent,

&result->warning_minutes)) {

/* Only elevate status when fault confidence already meets the warning threshold.

* Trend prediction alone cannot override a sub-threshold confidence reading. */

if (result->status == AI_STATUS_NORMAL &&

result->confidence_percent >= AI_WARNING_THRESHOLD_PERCENT) {

result->status = AI_STATUS_WARNING;

}

}

return 0;

}

int AI_ProcessChannelVoltage(uint16_t channel, float voltage_v, AI_Result *result)

{

AI_StateTemplate normal_template;

AI_Features current;

if (!g_ai.initialized) {

(void)AI_Init();

}

if (!ai_valid_channel(channel)) {

return -1;

}

if (ai_read_template(channel, AI_TEMPLATE_NORMAL, &normal_template) != 0 ||

!normal_template.valid) {

return -2;

}

ai_history_push_voltage(channel, voltage_v);

if (ai_current_features_from_history(channel, &normal_template, &current) != 0) {

return -3;

}

return AI_EvaluateChannel(channel, &current, result);

}

void AI_PrintResult(uint16_t channel, const AI_Result *result)

{

char line[160];

uint8_t board_id;

uint8_t board_channel;

int used;

if (result == 0 || result->status == AI_STATUS_NORMAL) {

return;

}

board_id = ai_board_id_from_channel(channel);

board_channel = ai_board_channel_from_channel(channel);

used = snprintf(line,

sizeof(line),

"%u_%02u:%s(%.0f%%)",

(unsigned)board_id,

(unsigned)board_channel,

g_type_names[result->type],

result->confidence_percent);

if (used < 0) {

return;

}

if (result->warning_type != AI_TEMPLATE_NORMAL && result->warning_minutes > 0) {

snprintf(line + used,

sizeof(line) - (size_t)used,

" | WARN:->%s ~%dmin",

g_type_names[result->warning_type],

result->warning_minutes);

}

strncat(line, "\r\n", sizeof(line) - strlen(line) - 1u);

AI_HW_UartWrite(line);

}

/* Acceptance/debug output: always prints, includes voltage and full prediction detail.

* Format: 1_05:V=1.235V,FAULT=Break(92%),WARN=Bend(78%),6min  */

static void ai_print_debug(uint16_t channel, float voltage_v, const AI_Result *result)

{

char line[160];

uint8_t board_id      = ai_board_id_from_channel(channel);

uint8_t board_channel = ai_board_channel_from_channel(channel);

int used;

used = snprintf(line, sizeof(line),

"%u_%02u:V=%.3fV,FAULT=%s(%.0f%%)",

(unsigned)board_id,

(unsigned)board_channel,

voltage_v,

g_type_names[result->type],

result->confidence_percent);

if (used < 0) {

return;

}

if (result->warning_type != AI_TEMPLATE_NORMAL && result->warning_minutes > 0) {

snprintf(line + used, sizeof(line) - (size_t)used,

",WARN=%s(%.0f%%),%dmin",

g_type_names[result->warning_type],

result->warning_confidence_percent,

result->warning_minutes);

} else {

strncat(line, ",WARN=none", sizeof(line) - strlen(line) - 1u);

}

strncat(line, "\r\n", sizeof(line) - strlen(line) - 1u);

AI_HW_UartWrite(line);

}

int AI_ProcessBoard(uint8_t board_id)

{

float   voltage[AI_CHANNELS_PER_BOARD];

uint8_t ch;

char    buf[32];

if (board_id < 1u || board_id > AI_BOARD_COUNT) {

return -1;

}

if (AI_HW_RS485_RequestBoard(board_id, voltage) != 0) {

g_board_fail_count[board_id - 1u]++;

if (g_board_online[board_id - 1u] && g_board_fail_count[board_id - 1u] >= 3u) {

snprintf(buf, sizeof(buf), "Board %u:OFFLINE\r\n", (unsigned)board_id);

AI_HW_UartWrite(buf);

g_board_online[board_id - 1u] = 0u;

}

return -2;

}

g_board_fail_count[board_id - 1u] = 0u;

if (!g_board_online[board_id - 1u]) {

snprintf(buf, sizeof(buf), "Board %u:ONLINE\r\n", (unsigned)board_id);

AI_HW_UartWrite(buf);

g_board_online[board_id - 1u] = 1u;

}

for (ch = 0; ch < AI_CHANNELS_PER_BOARD; ++ch) {

uint16_t channel = ai_make_channel(board_id, ch);

AI_Result result;

if (AI_ProcessChannelVoltage(channel, voltage[ch], &result) == 0) {

#if AI_DEBUG_MODE

ai_print_debug(channel, voltage[ch], &result);

#else

AI_PrintResult(channel, &result);

#endif

}

}

return 0;

}

void AI_ProcessAllBoards(void)

{

uint8_t board_id;

for (board_id = 1u; board_id <= AI_ACTIVE_BOARD_COUNT; ++board_id) {

(void)AI_ProcessBoard(board_id);

}

}

void AI_RunCalibration(void)

{

/* Must erase before writing; Flash bits can only go 0→1 via erase. */

AI_HW_UartWrite("CAL:Erasing Flash...\r\n");

if (AI_HW_FlashErase() != 0) {

AI_HW_UartWrite("CAL:Flash erase failed, abort\r\n");

return;

}

(void)AI_Init();

AI_HW_UartWrite("CAL:Normal - keep sample normal, press KEY0\r\n");

ai_wait_key_confirm();

(void)AI_CalibrateAll(AI_TEMPLATE_NORMAL);

AI_HW_UartWrite("CAL:Normal done\r\n");

AI_HW_UartWrite("CAL:Crack - switch sample, press KEY0\r\n");

ai_wait_key_confirm();

(void)AI_CalibrateAll(AI_TEMPLATE_CRACK);

AI_HW_UartWrite("CAL:Crack done\r\n");

AI_HW_UartWrite("CAL:Break - switch sample, press KEY0\r\n");

ai_wait_key_confirm();

(void)AI_CalibrateAll(AI_TEMPLATE_BREAK);

AI_HW_UartWrite("CAL:Break done\r\n");

AI_HW_UartWrite("CAL:Corrosion - switch sample, press KEY0\r\n");

ai_wait_key_confirm();

(void)AI_CalibrateAll(AI_TEMPLATE_CORROSION);

AI_HW_UartWrite("CAL:Corrosion done\r\n");

AI_HW_UartWrite("CAL:Bend - switch sample, press KEY0\r\n");

ai_wait_key_confirm();

(void)AI_CalibrateAll(AI_TEMPLATE_BEND);

AI_HW_UartWrite("CAL:Bend done\r\n");

(void)AI_SaveHeaderChecksum();

AI_HW_UartWrite("CAL:All done\r\n");

}

void AI_OneSecondTick(void)

{

if (!g_ai.initialized) {

(void)AI_Init();

}

if (AI_HW_KeyPressed()) {

AI_RunCalibration();

} else {

AI_ProcessAllBoards();

}

}

void AI_MainLoopTick(void)

{

AI_OneSecondTick();

}